GOOD LIQUID BEV CO

GMP / SQF SOP Supplement

Double-Seam Integrity · Label Control & Reconciliation · Master Sanitation Schedule

2011 51st Ave E, Unit 100, Palmetto, FL 34221

Revision 1.0  ·  Effective 2026-07-30  ·  Approved: Mike Krail — Owner / PCQI

These procedures close the documented-SOP gaps identified in the SQF Fundamentals review. Each pairs with a digital register in the CRM Daily GMP module.

# SOP — Double-Seam Integrity Inspection

| Document No. | SOP-DS-01 |
| --- | --- |
| Revision | 1.0 · Effective 2026-07-30 |
| Approved by | Mike Krail — Owner / PCQI |
| Review frequency | Annual, or on process/regulatory change |

## 1. Purpose

To verify that every can/end double seam produced meets integrity specifications, so that hermetically sealed, low-acid and acidified products remain commercially sterile and safe. A defective seam is a direct food-safety hazard (loss of hermetic seal → post-process contamination).

## 2. Scope

Applies to all seamed (double-seamed) container production on every seamer and seaming head at the Palmetto facility. Covers visual inspection at startup and at defined intervals, plus periodic seam teardown/micrometer measurement.

## 3. Definitions

- Double seam — the mechanical interlock of the can body flange and the end curl, formed in two roll operations (first- and second-operation rolls).

- Teardown — destructive seam examination (seam saw / micrometer / seam projector) measuring body hook, cover (end) hook, overlap, and tightness.

- Cutover / droop / vee / knock-down — visual seam defects indicating a first/second-operation or roll problem.

## 4. Responsibilities

- Line operator — performs and records visual checks each startup and interval; stops the line on any visual defect.

- Seaming technician / QA — performs scheduled teardowns and records micrometer dimensions; adjusts or locks out the seamer as needed.

- PCQI / owner — reviews the log daily, dispositions held product, and approves seamer release after corrective action.

## Procedure

- At each startup and at least every 30 minutes of run (and after any seamer adjustment or jam clear), pull a can from each operating head.

- Visually examine the seam around the full circumference for cutovers, droops, vees, sharp seams, false seams, or knock-down flanges. Record Pass/Fail.

- Perform a seam teardown at startup and at least once per 4-hour shift per seamer: measure body hook, cover hook, and overlap; assess tightness (wrinkle rating) and record the readings.

- Compare each dimension to the can/end supplier’s published seam specification for that container. Any value outside spec is a Fail.

- Enter all results in the Double-Seam Inspection Log (or the CRM Daily GMP entry). Sign the record.

## Monitoring

Visual: every startup and at ≤30-minute intervals per operating head. Teardown/micrometer: startup and ≥ once per 4-hour shift per seamer, and after any seamer change/adjustment.

## Acceptance criteria / limits

- Visual seam: no cutovers, droops, vees, sharp or false seams — must be Pass.

- Body hook, cover hook, overlap, tightness: within the container supplier’s seam specification for the specific can/end.

## Corrective action

- On any visual or dimensional Fail: stop the affected seamer/head immediately.

- Place all product produced back to the last documented good check on HOLD (DO NOT SHIP) and tag it.

- Adjust the seamer, re-seam a test can, and re-verify (visual + teardown) before resuming.

- PCQI dispositions held product (release, rework, or destroy) and records the disposition. Open a CAPA if the cause is systemic.

## Verification

PCQI reviews the double-seam log daily for completeness and out-of-spec trends. Seam micrometers/projectors are calibrated on a documented schedule. Records are retained ≥ 3 years (or per LACF/acidified requirement).

## Records

- Double-Seam Inspection Log (paper template herein or CRM register GMP-SEAM-001).

- Seam teardown measurement records; seamer adjustment / maintenance records.

- Hold tags and product disposition records for any Fail.

## References

- 21 CFR 113 (thermally processed low-acid foods) / 21 CFR 114 (acidified foods), as applicable.

- FDA Canned Foods — Principles of Thermal Process Control, Acidification, and Container Closure Evaluation.

- Container supplier double-seam specification sheets.

## Log template

Blank template shown below; the CRM Daily GMP module captures the same fields digitally (one entry fans out to every register).

| Date | Line / Run | Seamer / Head | Product / Lot | Visual OK | Body hook (in) | Cover hook (in) | Overlap (in) | Tightness OK | By / Initials |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Revision history

| Rev | Date | Description | Approved |
| --- | --- | --- | --- |
| 1.0 | 2026-07-30 | Initial issue — closes SQF documented-SOP gap. | MK |

# SOP — Label Control & Reconciliation

| Document No. | SOP-LC-01 |
| --- | --- |
| Revision | 1.0 · Effective 2026-07-30 |
| Approved by | Mike Krail — Owner / PCQI |
| Review frequency | Annual, or on process/regulatory change |

## 1. Purpose

To ensure the correct, approved label/artwork is applied to every product and that label usage reconciles at the end of each run — preventing mislabeled product (wrong allergen/ingredient statement, wrong net contents, wrong lot/date code) from reaching the field. Mislabeling is a leading cause of recalls.

## 2. Scope

Applies to all labels, shrink sleeves, printed cans/cartons, and applied date/lot codes for every finished-goods run at the Palmetto facility.

## 3. Definitions

- Artwork of record — the current, approved label version controlled in the specification file.

- Reconciliation — received quantity = applied (good) + damaged/scrapped + returned to stock.

## 4. Responsibilities

- Line operator — verifies the approved artwork/version before startup; counts and records labels received, applied, and scrapped.

- QA / PCQI — approves artwork versions, investigates any reconciliation variance, and authorizes finished-goods release.

## Procedure

- Before startup, pull the artwork of record for the scheduled product and confirm the label/sleeve on hand matches the approved version (brand, net contents, ingredient & allergen statement, UPC).

- Confirm the correct lot / date code is set and legible on a first-article unit.

- Record labels/sleeves received (issued to the line) for the run.

- At end of run, count applied/good units, damaged/scrapped labels, and any returned to stock.

- Reconcile: received must equal applied + scrapped + returned. Record Pass/Fail and any variance explanation.

- Destroy or quarantine obsolete/superseded label stock so it cannot be used.

## Monitoring

Artwork/version check and first-article code check every run at startup. Reconciliation at the end of every run.

## Acceptance criteria / limits

- Artwork/label version = approved version on file (Pass).

- Reconciliation gap = 0 unexplained units (received = applied + scrapped + returned).

## Corrective action

- Wrong or unapproved artwork: stop, remove incorrect labels from the line, load correct approved stock, verify first-article, then resume.

- Reconciliation variance: quarantine the run and investigate before release — a gap can indicate mislabeled units. PCQI dispositions and opens a CAPA.

- If suspect product already shipped, initiate the traceability/recall procedure.

## Verification

PCQI reviews label reconciliation records for every run; verifies obsolete artwork is removed. Records retained ≥ 3 years.

## Records

- Label / Artwork Reconciliation Log (template herein or CRM register GMP-LABEL-001).

- Approved artwork specification file with version history.

## References

- 21 CFR 101 (Food Labeling); FALCPA allergen labeling.

- SQF Food Safety Code — product identification, label reconciliation.

## Log template

Blank template shown below; the CRM Daily GMP module captures the same fields digitally (one entry fans out to every register).

| Date | Product / Brand | Lot / Date code | Artwork approved | Received (qty) | Applied (qty) | Scrapped (qty) | Reconciles | Variance note | By |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Revision history

| Rev | Date | Description | Approved |
| --- | --- | --- | --- |
| 1.0 | 2026-07-30 | Initial issue — closes SQF documented-SOP gap. | MK |

# Program — Master Sanitation Schedule (MSS)

| Document No. | SOP-MSS-01 |
| --- | --- |
| Revision | 1.0 · Effective 2026-07-30 |
| Approved by | Mike Krail — Owner / PCQI |
| Review frequency | Annual, or on process/regulatory change |

## 1. Purpose

To define the scheduled cleaning of all equipment, utensils, and facility areas beyond routine pre-operational cleaning, so that food-contact and non-food-contact surfaces are maintained in a sanitary condition and cross-contamination / allergen carryover is controlled.

## 2. Scope

Applies to all production equipment, tanks, fillers, seamers, conveyors, floors, drains, walls, overheads, and support areas at the Palmetto facility.

## 3. Definitions

- MSS — the master list of periodic (weekly/monthly/quarterly) cleaning tasks with assigned frequency, responsibility, and verification.

- Pre-op — the daily pre-operational sanitation verified before startup (see register GMP-PREOP-001); the MSS covers the deeper periodic tasks.

## 4. Responsibilities

- Sanitation lead — executes scheduled tasks, initials completion, flags anything needing maintenance.

- PCQI / owner — verifies completion (visual/ATP as applicable), signs off, and reschedules missed tasks.

## Procedure

- Maintain the Master Sanitation Schedule (grid below) listing each area/equipment, cleaning method & chemical, and frequency.

- Perform each task on its due date; initial and date the schedule on completion.

- Verify effectiveness by visual inspection, and by ATP/swab for food-contact surfaces where required.

- A missed or failed task is rescheduled immediately and, if it affects a startup, the affected line is not run until pre-op passes.

- Record chemical concentrations/titrations where applicable.

## Monitoring

Each task monitored on its scheduled frequency; pre-op verification (GMP-PREOP-001) confirms readiness daily before startup.

## Acceptance criteria / limits

- All scheduled tasks completed by due date; food-contact verification passes (visual and, where used, ATP ≤ site limit).

## Corrective action

- Missed task: complete as soon as identified, record, and reschedule; assess whether any product was made on an unclean line.

- Failed verification: re-clean and re-verify before use; open a CAPA if recurring.

## Verification

PCQI reviews the completed MSS weekly for completeness; ATP/swab records reviewed against limits. Records retained ≥ 3 years.

## Records

- Master Sanitation Schedule grid (herein / CRM CIP logs).

- Pre-Operational Sanitation Verification (GMP-PREOP-001).

- ATP / environmental swab results; chemical titration records.

## References

- 21 CFR 117 Subpart B (CGMP) — sanitation.

- SQF Food Safety Code — cleaning and sanitation.

## Log template

Blank template shown below; the CRM Daily GMP module captures the same fields digitally (one entry fans out to every register).

| Area / Equipment | Cleaning method & chemical | Frequency | Responsible | Done (init/date) | Verified (PCQI) |
| --- | --- | --- | --- | --- | --- |

## Revision history

| Rev | Date | Description | Approved |
| --- | --- | --- | --- |
| 1.0 | 2026-07-30 | Initial issue — closes SQF documented-program gap. | MK |